4. Theory

This circuit is a resistive network with two voltage inputs, VA and VB, each connected through a resistor to a common node, and a third resistor connecting that node to ground. The node voltage, VSUM, is determined entirely by the values of the three resistors and the two input voltages — not by simple arithmetic addition.

At the node, three currents meet: one flowing in from VA, one flowing in from VB, and one flowing out to ground. By Kirchhoff's Current Law, the sum of currents entering the node must equal the current leaving it. Taking the two input resistors as equal to R, the grounding resistor as R′, and assuming the measuring instrument draws negligible current (high input impedance):

(VA − VSUM)/R + (VB − VSUM)/R = VSUM/R′

Rearranging:

VSUM = (VA + VB) / (2 + R/R′)

This is the general behaviour of the network: VSUM is a weighted combination of VA and VB, with the weighting set by the ratio of the input resistors to the grounding resistor. In the special case where the grounding resistor R′ is much larger than the input resistors R (R/R′ → 0), the ground path draws comparatively little current, and the expression simplifies to:

VSUM ≈ (VA + VB)/2

In other words, the node settles at the average of the two input voltages rather than their sum. This matches the measured data exactly (e.g. VA = 5 V, VB = 10 V gives VSUM = 7.5 V, not 15 V), confirming that the circuit is a passive resistive averaging network, not a true adder.

The accuracy of this relationship depends on the internal impedance of the voltmeter used to read VSUM — a point the manual itself raises, describing the output node as being "left floating or terminated depending on measurement method and internal impedance of voltmeter." A low-impedance meter would introduce an additional current path to ground and pull VSUM below the ideal average; the high-impedance ("floating") measurement is what allows the clean VSUM ≈ (VA + VB)/2 relationship to be observed. The manual also frames the circuit as "claimed to perform function SUM = A + B" (Figure 2) — a claim the derivation above, and the measured results, show to be incorrect.

DIAGRAMS: Check the manual

5. Table of Values

| VA (V) | VB (V) | VSUM Measured (V) |
| --- | --- | --- |
| 0 | 0 | 0.00 |
| 0 | 5 | 2.50 |
| 0 | 10 | 5.00 |
| 5 | 5 | 5.00 |
| 5 | 10 | 7.50 |
| 10 | 10 | 10.00 |

6. Procedure

- Connect the two DC power supplies to inputs VA and VB respectively.

- Connect the digital multimeter across the output node to measure VSUM.

- Systematically vary VA and VB across the six combinations listed in the table above.

- Record the resulting VSUM for each combination.

- From the results, determine the relationship between VA, VB and VSUM, and state whether the system is analogue or digital.

7. Precautions

- Set each DC supply to the intended voltage before connecting it to the circuit.

- Verify resistor values with the multimeter before inserting them, to avoid using an incorrect value.

- Do not exceed the 0.5 W power rating of the resistors.

- Observe correct polarity when connecting the DC supplies.

- Allow the output reading to settle before recording it.

- Switch off the power supplies before altering any wiring.

8. Discussion

The measured results follow VSUM = (VA + VB)/2 exactly for all six test points, rather than VSUM = VA + VB as claimed by the block diagram in Figure 2. For example, with VA = 5 V and VB = 10 V, true summation would give 15 V, but the measured VSUM was only 7.50 V — precisely the average of the two inputs. This confirms the theoretical derivation: because the grounding resistor is large relative to the input resistors, the circuit behaves as a resistive averager rather than an adder.

Because VSUM varies continuously and proportionally with VA and VB over the full 0–10 V range, rather than switching between two discrete states, the circuit is an analogue system, not a digital one. It has no defined logic thresholds and no discrete HIGH/LOW output levels.

9. Conclusion

The tested circuit does not perform true arithmetic summation. It is a passive analogue averaging circuit obeying VSUM = (VA + VB)/2. The correct block diagram for its actual operation is "AVERAGE = (A + B)/2" rather than "SUM = A + B." The system is analogue, since its output is a continuous function of two continuous input voltages.

EXPERIMENT 2: LOGIC CIRCUITS

1. Aim

To investigate three discrete-component logic circuits (two diode-resistor circuits and one transistor circuit) and to identify the Boolean function performed by each.

2. Objective

- To construct Circuits A, B and C using diodes, resistors and an NPN transistor.

- To measure the output voltage of each circuit for every input combination.

- To classify each output as logic 0 (0–1 V) or logic 1 (4–5 V).

- To determine the Boolean function of each circuit and to evaluate what happens if several such circuits are cascaded in series.

3. Apparatus

- 5 V DC power supply

- 1 kΩ resistors (pull-up / pull-down, one per diode circuit)

- 10 kΩ resistor (transistor base resistor)

- Low-power silicon diodes (two per diode circuit)

- One NPN transistor (BC108)

- Voltmeter (analogue or digital)

- Prototyping board and connecting wires

4. Theory

This experiment looks at how simple combinations of diodes and a transistor can mimic the basic digital logic gates — AND, OR, and NOT — without using dedicated logic-gate ICs. Logic itself works with just two states: TRUE (HIGH, or logic 1) and FALSE (LOW, or logic 0). In positive logic, the more positive voltage represents TRUE and the less positive voltage represents FALSE — for this experiment, a reading of 0–1 V is treated as logic 0, and 4–5 V as logic 1.

Circuit A — Diode AND gate: Inputs A and B each feed into a common node through a diode, and a 1 kΩ resistor pulls that node up toward +5 V. If either input is LOW, its diode conducts and clamps the node close to that input's low voltage, holding the output LOW. The output only rises HIGH when both A and B are HIGH — neither diode conducts, and the pull-up resistor takes the node to +5 V. This gives OQ = A AND B.

Circuit B — Diode OR gate: Inputs A and B again feed a common node through diodes, but here a 1 kΩ resistor pulls the node down toward 0 V. If either input is HIGH, its diode conducts and pulls the node up close to that input's voltage. The output is LOW only when both A and B are LOW. This gives OQ = A OR B.

Circuit C — Transistor inverter (NOT gate): Input A drives the base of an NPN transistor (BC108) through a 10 kΩ resistor. The collector is pulled up to +5 V through a 1 kΩ resistor, and the emitter is grounded; the output is taken at the collector. When A is HIGH, the transistor saturates and pulls the collector LOW. When A is LOW, the transistor is OFF (cut-off), and the collector is pulled HIGH by the 1 kΩ resistor. This gives OQ = NOT A.

DIAGRAMS: Check the manual

5. Table of Values

Circuit A (AND):

| A | B | OQ (A) |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 0 |
| 1 | 0 | 0 |
| 1 | 1 | 1 |

Circuit B (OR):

| A | B | OQ (B) |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 1 |
| 1 | 0 | 1 |
| 1 | 1 | 1 |

Circuit C (NOT):

| A | OQ (C) |
| --- | --- |
| 0 | 1 |
| 1 | 0 |

6. Procedure

- Connect each circuit (A, B, C) as shown in the diagram, using the prototyping board.(I advice maybe drawing a detailed diagram of the half adder you built or the one in the manual and then referencing it the procedures)

- Use the voltmeter to measure the output voltage at each circuit's designated output node.

- Vary the input(s) through all combinations and complete a truth table for each circuit.

- Classify readings of 0–1 V as logic 0 and 4–5 V as logic 1.

7. Precautions

- Observe correct diode polarity — a reversed diode will not conduct as intended.

- Do not exceed the transistor's maximum collector current or power rating.

- Confirm all resistor values with a multimeter before inserting them.

- Avoid shorting +5 V directly to ground.

- Allow the output to settle before taking each voltmeter reading.

- Handle the BC108 transistor by its leads and avoid excessive heat near the package.

8. Discussion

The three circuits demonstrate that basic Boolean functions (AND, OR, NOT) can be implemented with discrete diodes and a transistor rather than dedicated logic-gate ICs. Diode gates exploit the one-way conduction of diodes, while the transistor gate uses the switching (saturation/cut-off) behaviour of a BJT to invert and restore the logic level.

A practical limitation of diode logic is that each conducting diode introduces a forward voltage drop of approximately 0.6–0.7 V. If several diode gates (Circuits A and/or B) are connected in series without an intervening restoring stage, this drop accumulates: the HIGH level droops and the LOW level rises with each stage. After a few cascaded stages, the signal can fall outside the 0–1 V / 4–5 V thresholds used to define logic 0 and 1, causing logic errors. A transistor stage such as Circuit C, which actively switches and restores full-swing output levels, is normally needed between diode-logic stages to prevent this degradation.

9. Conclusion

Circuit A implements the AND function, Circuit B implements the OR function, and Circuit C implements the NOT (inverter) function. Diode-resistor logic gates cannot be cascaded indefinitely in series without signal degradation, because of the cumulative diode voltage drop; periodic restoration using an active device (e.g. a transistor stage) is required.

Experiment 3: Half Adder Circuit

1. Aim

To construct and verify the operation of a Half Adder circuit built from basic logic gates.

2. Objective

- To understand how a Half Adder produces SUM and CARRY outputs from two single-bit inputs, A and B.

- To build the circuit shown in Figure 3 (Half Adder) on a breadboard using standard logic-gate ICs.

- To verify the circuit's operation by completing a truth table.

- To retain the constructed circuit for use in the Full Adder experiment (Experiment 4).

3. Apparatus

- 5 V DC power supply

- Logic gate ICs (AND and XOR-equivalent gates)

- Prototyping board

- Logic probe

- Connecting wires

4. Theory

A Half Adder is the simplest arithmetic circuit in digital electronics — it adds two single bits, A and B, and produces two outputs: SUM and CARRY.

To see why two outputs are needed, consider adding two binary digits by hand, the same way you would add two decimal digits:

| A | B | Result |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 1 |
| 1 | 0 | 1 |
| 1 | 1 | 10 (binary) |

The first three cases fit in a single bit. But 1 + 1 = 10 in binary — a two-digit result. The Half Adder splits this result into two separate single-bit signals: SUM, which holds the least-significant digit of the result, and CARRY, which holds the digit that overflows into the next column (just as, in decimal addition, a carry is passed to the next column when a column sums to 10 or more).

Comparing this table with the truth tables of basic logic gates shows that:

- SUM is 1 only when exactly one of A, B is 1 — this is the definition of the Exclusive-OR (XOR) function: SUM = A ⊕ B.

- CARRY is 1 only when both A and B are 1 — this is the definition of the AND function: CARRY = A · B.

So the Half Adder can be built entirely from an XOR gate and an AND gate operating on the same two inputs, in parallel. Since dedicated XOR gates aren't always available, the XOR function itself can be built from more basic AND/OR/NOT gates — which is why Figure 3 shows two AND gates and an inverting stage used together to realise the SUM output, while a separate, direct AND gate produces CARRY.

The main limitation of the Half Adder is that it only has two inputs — it has no way to accept a carry bit coming in from a previous (less significant) column of addition. This means it can only be used to add the very first (least-significant) pair of bits in a multi-bit sum; every other column needs a carry-in as well.

DIAGRAMS: Check the manual

6. Procedure

- Wire the logic gate ICs on the breadboard as shown in the diagram, and power them with 5 V. (I advice maybe drawing a detailed diagram of the half adder you built or the one in the manual and then referencing it the procedures)

- Apply all four combinations of A and B using switches.

- Observe the SUM and CARRY outputs using a logic probe or LED indicators.

- Record the results in the truth table and retain the circuit for Experiment 4.

7. Precautions

- Switch off the power supply before changing any wiring.

- Ensure IC pins are correctly oriented (align the pin-1 notch) before insertion.

- Do not leave unused gate inputs floating; tie them to a defined logic level.

- Use current-limiting resistors for any LED indicators.

- Verify the +5 V and ground rails before powering the board.

8. Discussion

The measured truth table matched the theoretical Half Adder function exactly: SUM followed the XOR pattern and CARRY followed the AND pattern for all four input combinations. A key limitation of the Half Adder is that it has no carry-in input, so it can only add two bits at a time and cannot, on its own, be chained to add multi-bit numbers beyond the least-significant bit position.

9. Conclusion

The constructed circuit correctly implements the Half Adder Boolean functions, SUM = A ⊕ B and CARRY = A · B, matching the theoretical truth table. This circuit forms the basic building block used in Experiment 4 to construct a Full Adder.

Experiment 4: Full Adder Circuit

1. Aim

To design, implement and verify a Full Adder circuit that adds three input bits (A, B and Cin) and produces a SUM and a CARRY-OUT (Cout).

2. Objective

- To design and implement a Full Adder using basic logic gates (AND, OR, XOR).

- To understand the function of a Full Adder in performing binary addition of three input bits: A, B, and Cin (carry-in).

- To analyse the outputs: Sum (S), the result of the addition; and Carry-out (Cout), the carry that can be passed to the next stage of a multi-bit adder.

- To develop skills in digital circuit construction and logic analysis using breadboards / prototyping tools.

3. Apparatus

- 5 V DC power supply

- Two previously-constructed Half Adder circuits (from Experiment 3), or equivalent gate ICs

- Assorted logic gates: AND (7408), OR (7432), XOR (7486)

- 1 kΩ resistors

- Prototyping board, logic probe, switches (for inputs A, B, Cin) and LEDs (for SUM and CARRY outputs)

4. Theory

The Half Adder built in Experiment 3 has a limitation: it can only add two bits, A and B, with no way to include a carry coming in from a previous column of addition. In practice, adding numbers that are more than one bit wide requires every column (except the very first) to add three values together — the two bits in that column, plus whatever carry was generated by the column before it. A Full Adder solves this by taking a third input, Cin (carry-in), alongside A and B.

Working through the addition of three bits by hand shows the pattern the Full Adder must implement:

| A | B | Cin | Result |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 1 |
| 0 | 1 | 0 | 1 |
| 0 | 1 | 1 | 10 |
| 1 | 0 | 0 | 1 |
| 1 | 0 | 1 | 10 |
| 1 | 1 | 0 | 10 |
| 1 | 1 | 1 | 11 |

As before, a two-bit result must be split into two single-bit outputs: Sum, the least-significant digit, and Cout, the carry passed on to the next stage.

Comparing this against known logic behaviour:

- Sum is 1 whenever an odd number of the three inputs are 1 — this is the three-input Exclusive-OR function: Sum = A ⊕ B ⊕ Cin.

- Cout is 1 whenever at least two of the three inputs are 1 — this is captured by ORing together every pair of inputs ANDed together: Cout = (A AND B) OR (B AND Cin) OR (A AND Cin).

A convenient way to build this is to reuse two Half Adders (Figure 4), since a Half Adder already computes SUM and CARRY for two inputs:

- The first Half Adder adds A and B, giving an intermediate SUM and an intermediate CARRY.

- The second Half Adder adds that intermediate SUM to Cin, giving the final Sum and a second CARRY.

- Because either half-adder stage could generate a carry (but never both at once), their two CARRY outputs are combined with an OR gate to produce the final Cout.

This two-Half-Adder-plus-OR structure and the direct gate-level equations above are two different ways of building the exact same Boolean function — they should produce identical truth tables.

DIAGRAMS: Check the manual

5. Table of Values

| A | B | Cin | Sum | Cout |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 1 | 0 |
| 0 | 1 | 0 | 1 | 0 |
| 0 | 1 | 1 | 0 | 1 |
| 1 | 0 | 0 | 1 | 0 |
| 1 | 0 | 1 | 0 | 1 |
| 1 | 1 | 0 | 0 | 1 |
| 1 | 1 | 1 | 1 | 1 |

6. Procedure

Method 1 — using two Half Adders (Figure 4):

- Feed A and B into the first Half Adder, obtaining an intermediate SUM and CARRY.

- Feed the intermediate SUM and Cin into the second Half Adder, obtaining the final SUM and a second CARRY.

- OR the two CARRY signals together to obtain Cout.

Method 2 — using discrete logic gates:

- Connect the ICs to the breadboard and power them with 5 V.

- Use XOR gates (7486) to compute A ⊕ B, then ⊕ Cin, for the SUM.

- Use three AND gates (7408) to compute A AND B, B AND Cin, and A AND Cin.

- Use two OR gates (7432) to combine the AND results and produce Cout.

- Connect the inputs A, B and Cin through switches and resistors.

- Connect LEDs (through 1 kΩ resistors) to the SUM and Cout outputs for visualisation.

For either method, apply all eight combinations of A, B and Cin and record SUM and Cout.

7. Precautions

- Power down the circuit before rewiring between the two construction methods.

- Verify gate pin-outs (7486, 7408, 7432) against the datasheet before wiring.

- Do not leave any carry-chain input floating.

- Use current-limiting resistors for all LED indicators.

- Confirm that each Half Adder sub-circuit (Experiment 3) functions correctly before combining them.

8. Discussion

Both realisations of the Full Adder produced the same 8-row truth table, confirming that the two-Half-Adder-plus-OR structure and the direct gate-level equations (Sum = A⊕B⊕Cin, Cout = AB + BCin + ACin) are logically equivalent.

Comparing this result with the Half Adder truth table from Experiment 3 shows that the Full Adder reduces to the Half Adder whenever Cin = 0: for those rows, Sum and Cout in this table match SUM and CARRY in the Half Adder table exactly. The Full Adder is therefore a generalisation of the Half Adder that additionally accepts a carry-in, which is what allows multiple Full Adders to be chained together (ripple-carry) to build a multi-bit binary adder — a core building block of an ALU (Arithmetic Logic Unit) in a processor such as the 8085/Z80 referenced in Experiment 1.

9. Conclusion

The Full Adder circuit correctly sums three input bits, producing Sum = A ⊕ B ⊕ Cin and Cout = AB + BCin + ACin. It was successfully realised in two equivalent ways — by cascading two Half Adders with an OR gate, and directly from XOR/AND/OR gates — both of which produced identical, theoretically-correct truth tables. This confirms the Full Adder as the fundamental building block for constructing multi-bit binary adders and ALUs.