ELECTRICAL AND LOGIC CIRCUITS

Laboratory Report

Experiments 1 – 4: 8085/Z80 Application, Logic Circuits, Half Adder, Full Adder

Name: ____________________________

Registration Number: ______________________

Course: Electrical/Computer Engineering — ELA Laboratory

Date: ____________________________

# Experiment 1: Electrical and Logic Circuits, 8085/Z80 Application

## 1. Aim

To investigate the two-input resistor network shown in Figure 1 (the “SUM” circuit) and to determine the actual mathematical relationship between the input voltages VA, VB and the measured output voltage VSUM.

## 2. Objective

- To construct the circuit using two 100 kΩ input resistors and a 100 kΩ load resistor on a prototyping board.

- To measure VSUM for six combinations of VA and VB using a digital multimeter.

- To determine whether the circuit truly performs the arithmetic function SUM = A + B, as claimed in Figure 2, or some other function.

- To classify the operation of the circuit as analogue or digital, with justification.

## 3. Apparatus

- Two 0–10 V DC power supplies

- Digital multimeter

- Two 100 kΩ (0.5 W) resistors

- One 100 kΩ (0.5 W) resistor

- Prototyping (bread)board

- Connecting wires

## 4. Theory / Diagram

The circuit (Figure 1) has two input resistors, one from VA and one from VB, meeting at a common node. A third 100 kΩ resistor connects this node to ground, and the node voltage is measured as VSUM using a high-impedance digital multimeter (which draws negligible current).

Applying Kirchhoff's Current Law at the common node, with the input resistors equal to R and the grounding resistor equal to R′:

(VA − VSUM)/R + (VB − VSUM)/R = VSUM/R′

Rearranging: VSUM = (VA + VB) / (2 + R/R′)

When the grounding resistor R′ is much larger than the input resistors (R/R′ → 0), this reduces to VSUM ≈ (VA + VB)/2. This is the behaviour confirmed by the measurements in Section 5 below: the node does not add VA and VB arithmetically — it produces their average. The circuit is therefore a passive resistive averaging (voltage‑mixing) network, not a true adder.

## 5. Table of Values

| VA (V) | VB (V) | VSUM Measured (V) |
| --- | --- | --- |
| 0 | 0 | 0.00 |
| 0 | 5 | 2.50 |
| 0 | 10 | 5.00 |
| 5 | 5 | 5.00 |
| 5 | 10 | 7.50 |
| 10 | 10 | 10.00 |

## 6. Procedure

- Connect the two DC power supplies to inputs VA and VB respectively.

- Connect the digital multimeter across the output node to measure VSUM.

- Systematically vary VA and VB across the six combinations listed in the table.

- Record the resulting VSUM for each combination.

- From the results, determine the relationship between VA, VB and VSUM, and state whether the system is analogue or digital.

## 7. Precautions

- Set each DC supply to the intended voltage before connecting it to the circuit.

- Verify resistor values with the multimeter before inserting them, to avoid using an incorrect value.

- Do not exceed the 0.5 W power rating of the resistors.

- Observe correct polarity when connecting the DC supplies.

- Allow the output reading to settle before recording it.

- Switch off the power supplies before altering any wiring.

## 8. Discussion

The measured results follow VSUM = (VA + VB)/2 exactly for all six test points, rather than VSUM = VA + VB as claimed by the block diagram in Figure 2. For example, with VA = 5 V and VB = 10 V, true summation would give 15 V, but the measured VSUM was only 7.50 V — precisely the average of the two inputs. This confirms the theoretical derivation in Section 4: because the grounding resistor is large relative to the effective source resistance at the node, the circuit behaves as a resistive averager rather than an adder.

Because VSUM varies continuously and proportionally with VA and VB over the full 0–10 V range, rather than switching between two discrete states, the circuit is an analogue system, not a digital one. It has no defined logic thresholds and no discrete HIGH/LOW output levels.

## 9. Conclusion

The tested circuit does not perform true arithmetic summation. It is a passive analogue averaging circuit obeying VSUM = (VA + VB)/2. The correct block diagram for its actual operation is “AVERAGE = (A + B)/2” rather than “SUM = A + B.” The system is analogue, since its output is a continuous function of two continuous input voltages.

## Graph

The chart below compares the measured VSUM against the averaging prediction (VA + VB)/2 and against the hypothetical true-summation output (VA + VB), showing that the measured data coincides with the averaging curve.

# Experiment 2: Logic Circuits (Diode–Transistor Logic)

## 1. Aim

To investigate three discrete-component logic circuits (two diode-resistor circuits and one transistor circuit) and to identify the Boolean function performed by each.

## 2. Objective

- To construct Circuits A, B and C (Figure 3) using diodes, resistors and an NPN transistor.

- To measure the output voltage of each circuit for every input combination.

- To classify each output as logic 0 (0–1 V) or logic 1 (4–5 V).

- To determine the Boolean function of each circuit and to evaluate what happens if several such circuits are cascaded in series.

## 3. Apparatus

- 5 V DC power supply

- 1 kΩ resistors (pull-up / pull-down, one per diode circuit)

- 10 kΩ resistor (transistor base resistor)

- Low-power silicon diodes (two per diode circuit)

- One NPN transistor (BC108)

- Voltmeter (analogue or digital)

- Prototyping board and connecting wires

## 4. Theory / Diagram

### Circuit A — Diode AND gate

Inputs A and B feed a common node through diodes, with a 1 kΩ resistor pulling the node up to +5 V. If either input is LOW, its diode conducts and clamps the node close to that input's voltage, so the output stays LOW. The output only rises HIGH when both A and B are HIGH (no diode conducts, and the pull-up resistor takes the node to +5 V). This realizes OQ = A AND B.

### Circuit B — Diode OR gate

Inputs A and B feed a common node through diodes, with a 1 kΩ resistor pulling the node down to 0 V. If either input is HIGH, its diode conducts and pulls the node up close to that input's voltage. The output is LOW only when both A and B are LOW. This realizes OQ = A OR B.

### Circuit C — Transistor inverter (NOT gate)

Input A drives the base of an NPN transistor (BC108) through a 10 kΩ resistor; the collector is pulled up to +5 V through 1 kΩ, and the emitter is grounded. When A is HIGH, the transistor saturates and pulls the collector (output) LOW. When A is LOW, the transistor is OFF and the collector is pulled HIGH by the 1 kΩ resistor. This realizes OQ = NOT A.

## 5. Table of Values

Circuit A (AND):

| A | B | OQ (A) |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 0 |
| 1 | 0 | 0 |
| 1 | 1 | 1 |

Circuit B (OR):

| A | B | OQ (B) |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 1 |
| 1 | 0 | 1 |
| 1 | 1 | 1 |

Circuit C (NOT):

| A | OQ (C) |
| --- | --- |
| 0 | 1 |
| 1 | 0 |

## 6. Procedure

- Connect each circuit (A, B, C) as shown in Figure 3, using the prototyping board.

- Use the voltmeter to measure the output voltage at each circuit's designated output node.

- Vary the input(s) through all combinations and complete a truth table for each circuit.

- Classify readings of 0–1 V as logic 0 and 4–5 V as logic 1.

## 7. Precautions

- Observe correct diode polarity — a reversed diode will not conduct as intended.

- Do not exceed the transistor's maximum collector current or power rating.

- Confirm all resistor values with a multimeter before inserting them.

- Avoid shorting +5 V directly to ground.

- Allow the output to settle before taking each voltmeter reading.

- Handle the BC108 transistor by its leads and avoid excessive heat near the package.

## 8. Discussion

The three circuits demonstrate that basic Boolean functions (AND, OR, NOT) can be implemented with discrete diodes and a transistor rather than dedicated logic-gate ICs. Diode gates exploit the one-way conduction of diodes, while the transistor gate uses the switching (saturation/cut-off) behaviour of a BJT to invert and restore the logic level.

A practical limitation of diode logic is that each conducting diode introduces a forward voltage drop of approximately 0.6–0.7 V. If several diode gates (Circuits A and/or B) are connected in series without an intervening restoring stage, this drop accumulates: the HIGH level droops and the LOW level rises with each stage. After a few cascaded stages the signal can fall outside the 0–1 V / 4–5 V thresholds used to define logic 0 and 1, causing logic errors. A transistor stage such as Circuit C, which actively switches and restores full-swing output levels, is normally needed between diode-logic stages to prevent this degradation.

## 9. Conclusion

Circuit A implements the AND function, Circuit B implements the OR function, and Circuit C implements the NOT (inverter) function. Diode-resistor logic gates cannot be cascaded indefinitely in series without signal degradation, because of the cumulative diode voltage drop; periodic restoration using an active device (e.g. a transistor stage) is required.

# Experiment 3: Half Adder Circuit

## 1. Aim

To construct and verify the operation of a Half Adder circuit built from basic logic gates.

## 2. Objective

- To understand how a Half Adder produces SUM and CARRY outputs from two single-bit inputs, A and B.

- To build the circuit shown in Figure 3 (Half Adder) on a breadboard using standard logic-gate ICs.

- To verify the circuit's operation by completing a truth table.

- To retain the constructed circuit for use in the Full Adder experiment (Experiment 4).

## 3. Apparatus

- 5 V DC power supply

- Logic gate ICs (AND and XOR-equivalent gates)

- Prototyping board

- Logic probe

- Connecting wires

## 4. Theory / Diagram

A Half Adder adds two single bits, A and B, producing two outputs: SUM and CARRY. The SUM output is the Exclusive-OR (XOR) of the inputs, while the CARRY output is their logical AND:

SUM = A ⊕ B          CARRY = A · B

In Figure 3, this is realised using two AND gates and an inverting (“1”) stage arranged to produce the XOR function for SUM, together with a separate AND gate that produces CARRY directly from A and B.

## 5. Table of Values

| A | B | SUM | CARRY |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 0 | 1 | 1 | 0 |
| 1 | 0 | 1 | 0 |
| 1 | 1 | 0 | 1 |

## 6. Procedure

- Wire the logic gate ICs on the breadboard as shown in Figure 3, and power them with 5 V.

- Apply all four combinations of A and B using switches.

- Observe the SUM and CARRY outputs using a logic probe or LED indicators.

- Record the results in the truth table and retain the circuit for Experiment 4.

## 7. Precautions

- Switch off the power supply before changing any wiring.

- Ensure IC pins are correctly oriented (align the pin-1 notch) before insertion.

- Do not leave unused gate inputs floating; tie them to a defined logic level.

- Use current-limiting resistors for any LED indicators.

- Verify the +5 V and ground rails before powering the board.

## 8. Discussion

The measured truth table matched the theoretical Half Adder function exactly: SUM followed the XOR pattern and CARRY followed the AND pattern for all four input combinations. A key limitation of the Half Adder is that it has no carry-in input, so it can only add two bits at a time and cannot, on its own, be chained to add multi-bit numbers beyond the least-significant bit position.

## 9. Conclusion

The constructed circuit correctly implements the Half Adder Boolean functions, SUM = A ⊕ B and CARRY = A · B, matching the theoretical truth table. This circuit forms the basic building block used in Experiment 4 to construct a Full Adder.

# Experiment 4: Full Adder Circuit

## 1. Aim

To design, implement and verify a Full Adder circuit that adds three input bits (A, B and Cin) and produces a SUM and a CARRY-OUT (Cout).

## 2. Objective

- To design and implement a Full Adder using basic logic gates (AND, OR, XOR).

- To understand the function of a Full Adder in performing binary addition of three input bits: A, B, and Cin (carry-in).

- To analyse the outputs: Sum (S), the result of the addition; and Carry-out (Cout), the carry that can be passed to the next stage of a multi-bit adder.

- To develop skills in digital circuit construction and logic analysis using breadboards / prototyping tools.

## 3. Apparatus

- 5 V DC power supply

- Two previously-constructed Half Adder circuits (from Experiment 3), or equivalent gate ICs

- Assorted logic gates: AND (7408), OR (7432), XOR (7486)

- 1 kΩ resistors

- Prototyping board, logic probe, switches (for inputs A, B, Cin) and LEDs (for SUM and CARRY outputs)

## 4. Theory / Diagram

A Full Adder performs 1-bit binary addition of three bits, including a possible carry from a less-significant stage, producing a SUM and a possible carry to a more significant stage. Figure 4 shows it built by cascading two Half Adders and an OR gate: the first Half Adder adds A and B, producing an intermediate SUM and CARRY; the second Half Adder adds that intermediate SUM to Cin, producing the final SUM and a second CARRY; the two CARRY outputs are combined with an OR gate to give Cout.

Equivalently, at the gate level (as implemented directly with XOR, AND and OR gates):

Sum (S) = A ⊕ B ⊕ Cin

Carry-out (Cout) = (A AND B) OR (B AND Cin) OR (A AND Cin)

Both realisations — the two-Half-Adder-plus-OR structure and the direct XOR/AND/OR gate structure — are logically equivalent and should produce identical truth tables.

## 5. Table of Values

| A | B | Cin | Sum | Cout |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 1 | 0 |
| 0 | 1 | 0 | 1 | 0 |
| 0 | 1 | 1 | 0 | 1 |
| 1 | 0 | 0 | 1 | 0 |
| 1 | 0 | 1 | 0 | 1 |
| 1 | 1 | 0 | 0 | 1 |
| 1 | 1 | 1 | 1 | 1 |

## 6. Procedure

Method 1 — using two Half Adders (Figure 4):

- Feed A and B into the first Half Adder, obtaining an intermediate SUM and CARRY.

- Feed the intermediate SUM and Cin into the second Half Adder, obtaining the final SUM and a second CARRY.

- OR the two CARRY signals together to obtain Cout.

Method 2 — using discrete logic gates:

- Connect the ICs to the breadboard and power them with 5 V.

- Use XOR gates (7486) to compute A ⊕ B, then ⊕ Cin, for the SUM.

- Use three AND gates (7408) to compute A AND B, B AND Cin, and A AND Cin.

- Use two OR gates (7432) to combine the AND results and produce Cout.

- Connect the inputs A, B and Cin through switches and resistors.

- Connect LEDs (through 1 kΩ resistors) to the SUM and Cout outputs for visualisation.

For either method, apply all eight combinations of A, B and Cin and record SUM and Cout.

## 7. Precautions

- Power down the circuit before rewiring between the two construction methods.

- Verify gate pin-outs (7486, 7408, 7432) against the datasheet before wiring.

- Do not leave any carry-chain input floating.

- Use current-limiting resistors for all LED indicators.

- Confirm that each Half Adder sub-circuit (Experiment 3) functions correctly before combining them.

## 8. Discussion

Both realisations of the Full Adder produced the same 8-row truth table, confirming that the two-Half-Adder-plus-OR structure and the direct gate-level equations (Sum = A⊕B⊕Cin, Cout = AB + BCin + ACin) are logically equivalent.

Comparing this result with the Half Adder truth table from Experiment 3 shows that the Full Adder reduces to the Half Adder whenever Cin = 0: for those rows, Sum and Cout in this table match SUM and CARRY in the Half Adder table exactly. The Full Adder is therefore a generalisation of the Half Adder that additionally accepts a carry-in, which is what allows multiple Full Adders to be chained together (ripple-carry) to build a multi-bit binary adder, a core building block of an ALU (Arithmetic Logic Unit) in a processor such as the 8085/Z80 referenced in Experiment 1.

## 9. Conclusion

The Full Adder circuit correctly sums three input bits, producing Sum = A ⊕ B ⊕ Cin and Cout = AB + BCin + ACin. It was successfully realised in two equivalent ways — by cascading two Half Adders with an OR gate, and directly from XOR/AND/OR gates — both of which produced identical, theoretically-correct truth tables. This confirms the Full Adder as the fundamental building block for constructing multi-bit binary adders and ALUs.

# Questions Asked in the Manual

### Experiment 1

- Determine the relationship between VA, VB and VSUM — VSUM = (VA + VB)/2 (an averaging relationship, not arithmetic summation).

- Draw a block diagram describing the actual operation of the system — the correct block should read “AVERAGE = (A + B)/2” in place of the claimed “SUM = A + B” block shown in Figure 2.

- State, giving reasons, whether the system is analogue or digital — Analogue, because VSUM varies continuously with VA and VB rather than switching between two discrete logic levels.

### Experiment 2

- Based on the results, identify the Boolean function of each circuit — Circuit A = AND, Circuit B = OR, Circuit C = NOT.

- If several of the circuits were connected in series, will there be a problem? — Yes: the diode forward-voltage drop accumulates across cascaded diode-logic stages, degrading the HIGH/LOW levels until they can fall outside the valid logic-1/logic-0 thresholds; a restoring (active/transistor) stage is needed between stages.

### Experiments 3 and 4

- Compare and comment on the Full Adder truth table versus the Half Adder truth table — the Full Adder truth table matches the Half Adder's SUM/CARRY values exactly whenever Cin = 0; the Full Adder is a generalisation of the Half Adder that additionally handles a carry-in, enabling cascaded multi-bit addition.

# What I've Learned

- A passive resistor network only performs true arithmetic addition under specific impedance conditions; here it instead performed averaging, showing the importance of verifying a circuit's claimed function experimentally rather than assuming it from a block diagram.

- Diode-resistor networks can realise basic AND/OR logic cheaply, but they lack the level-restoring property of active gates, so diode logic degrades when cascaded and needs periodic transistor buffering.

- A single BJT stage can realise a logic inversion (NOT) while simultaneously restoring signal levels, illustrating why real logic-gate ICs are built from transistors rather than diodes alone.

- The Half Adder (SUM = A⊕B, CARRY = A·B) and Full Adder (Sum = A⊕B⊕Cin, Cout = AB+BCin+ACin) illustrate how larger digital building blocks (adders, and eventually ALUs) are constructed by combining smaller logic blocks, and that the same Boolean function can be implemented by more than one equivalent gate structure.

- This progression, from simple resistor/diode circuits to combinational adders, mirrors how arithmetic and logic units inside real microprocessors (e.g. the 8085/Z80 referenced in Experiment 1) are built up from simple logic-block hierarchies.

# Special Questions

Not covered in this laboratory session, as indicated in the manual.